# Sapphire Professional

# Installing a new lens or changing the mounting:

Remove the 4 screws to take off the backplate.

New Lens:

Lift off the exteriour housing and exchange the lens.

Please make sure that the two little noses at lower side of the lens-rim have to go into the notches of the dial.

When you slide over the exteriour housing, use your pointing-finger and thumb, to hold the interiour housing with the lens on the upper side and the membran-spring on the lower side firmly together.

The two noses on the upper side of the lens-rim have to go into the corresponding notches on the inside of the exteriour housing (there are four notches in the housing to allow a 90° position change as prefered).

Remark: You will be able to see the noses and the notches through the glas.

After you have double checked, that everything is aligned and in place, you can assemble

the backplate.

Changing the type of mounting:

No need to take off the lens, just keep the interiour housing with the lens on the upper side and the membran-spring on the lower side firmly together and rotate this package against

the exteriour housing as required.

Remark:

Any time when you change from elastic-mounting to O-ring (or back), you need to rotate

the interiour housing (complete with the dial and glas) 90° against the exteriour housing in order to have the 0-mark on the dial at the same place when you wear the altimeter on your hand.

The backplate with the elastic mounting, respectively with the O-ring-strap

(which should already be in place when the backplate is installed) can be assembled in either 180°-direction, to suite your prefered 0-mark dial-position of the altimeter).